#include <Wire.h>

#include <WiFi.h>

#include <HTTPClient.h>

#include <ArduinoJson.h>  // Add this library for better JSON handling

#include <time.h>         // Add this for NTP time functions

const char* ssid = "DESKTOP-9606";

const char* password = "123456789";

// Corrected Firebase configuration

const char* firebaseHost = "https://agrocare-ac440-default-rtdb.asia-southeast1.firebasedatabase.app/";

const char* firebaseAuth = "AIzaSyBwEEoSK8JL14BSeUEdNZbn2AOg9dbCkaY";

String receivedData = "";

bool newData = false;

// PROPERLY DECLARED SENSOR VARIABLES (this was the main issue)

float nitrogen = 0.0, phosphorus = 0.0, potassium = 0.0, temp = 0.0, hum = 0.0, ph = 0.0;

int ec = 0, soil = 0;

bool validData = false;  // Flag to check if we have valid sensor data

volatile bool i2cBusy = false;  // Used to avoid overlapping I2C and heavy operations

unsigned long lastUploadTime = 0;

const unsigned long uploadInterval = 5000;  // 5 seconds

// NTP Configuration

const char* ntpServer = "pool.ntp.org";

const long gmtOffset_sec = 19800;  // GMT+5:30 for India (5.5 hours * 3600 seconds)

const int daylightOffset_sec = 0;  // No daylight saving in India

void receiveEvent(int howMany) {

if (i2cBusy) return;  // Skip if we're currently uploading to Firebase

i2cBusy = true;

receivedData = "";

Serial.println("I2C interrupt triggered, expecting " + String(howMany) + " bytes");

while (Wire.available()) {

char c = Wire.read();

receivedData += c;

}

newData = true;

i2cBusy = false;

Serial.println("I2C Data Received (" + String(receivedData.length()) + " bytes): " + receivedData);

}

void setup() {

Serial.begin(115200);

delay(1000);

Serial.println("ESP32 starting...");

// Initialize I2C as slave with address 8

Wire.begin(8); // ESP32 I2C slave address

Wire.onReceive(receiveEvent);

Serial.println("I2C initialized as slave with address 8");

// Connect to WiFi

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println();

Serial.println("Connected to WiFi!");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

// Initialize NTP

Serial.println("Initializing NTP...");

configTime(gmtOffset_sec, daylightOffset_sec, ntpServer);

// Wait for time to be set

Serial.print("Waiting for NTP time sync");

time_t now = time(nullptr);

while (now < 8 * 3600 * 2) {

delay(500);

Serial.print(".");

now = time(nullptr);

}

Serial.println();

struct tm timeinfo;

if (getLocalTime(&timeinfo)) {

Serial.println("Time synchronized!");

Serial.println(&timeinfo, "Current time: %A, %B %d %Y %H:%M:%S");

} else {

Serial.println("Failed to obtain time");

}

Serial.println("ESP32 ready to receive I2C data...");

}

void parseReceivedData() {

if (receivedData.length() > 0) {

// Parse only 7 values: N, P, K, Temp, Humidity, pH, SoilMoisture

int parseResult = sscanf(receivedData.c_str(), "%f,%f,%f,%f,%f,%f,%d",

&nitrogen, &phosphorus, &potassium,

&temp, &hum, &ph, &soil);

if (parseResult == 7) {

validData = true;  // Mark that we have valid data

Serial.println("---- Parsed Sensor Data ----");

Serial.println("Nitrogen: " + String(nitrogen));

Serial.println("Phosphorus: " + String(phosphorus));

Serial.println("Potassium: " + String(potassium));

Serial.println("Temperature: " + String(temp));

Serial.println("Humidity: " + String(hum));

Serial.println("pH: " + String(ph));

Serial.println("Soil Moisture: " + String(soil));

Serial.println("--------------------------------");

} else {

Serial.println("❌ Error parsing data. Expected 7 values, got: " + String(parseResult));

Serial.println("Raw data: " + receivedData);

validData = false;

}

}

}

String getTimestamp() {

// Get current time in milliseconds since boot

unsigned long currentTime = millis();

// You can also use NTP time if you want real timestamps

// For now, using millis() as a simple timestamp

return String(currentTime);

}

String getEpochTimestamp() {

// Get real epoch timestamp from NTP

time_t now = time(nullptr);

return String(now);

}

String getCurrentDate() {

// Get real current date from NTP

struct tm timeinfo;

if (!getLocalTime(&timeinfo)) {

Serial.println("Failed to obtain time for date");

return "2025-01-01"; // fallback

}

char dateStr[11];

strftime(dateStr, sizeof(dateStr), "%Y-%m-%d", &timeinfo);

return String(dateStr);

}

String getCurrentDateTime() {

// Get real current date and time from NTP

struct tm timeinfo;

if (!getLocalTime(&timeinfo)) {

Serial.println("Failed to obtain time for datetime");

return "2025-01-01 00:00:00"; // fallback

}

char dateTimeStr[20];

strftime(dateTimeStr, sizeof(dateTimeStr), "%Y-%m-%d %H:%M:%S", &timeinfo);

return String(dateTimeStr);

}

void uploadToFirebase() {

if (WiFi.status() != WL_CONNECTED) {

Serial.println("WiFi not connected!");

return;

}

// Only upload if we have valid sensor data

if (!validData) {

Serial.println("No valid sensor data to upload yet.");

return;

}

HTTPClient http;

// Upload current sensor data to /sensors.json

String url = String(firebaseHost) + "/sensors.json?auth=" + firebaseAuth;

Serial.println("Uploading current data to: " + url);

http.begin(url);

http.addHeader("Content-Type", "application/json");

// Create JSON in the exact format you specified

StaticJsonDocument<512> doc;

// NPK section

JsonObject npk = doc.createNestedObject("NPK");

npk["N"] = nitrogen;

npk["P"] = phosphorus;

npk["K"] = potassium;

npk["PH"] = ph;

// dht11 section

JsonObject dht11 = doc.createNestedObject("dht11");

dht11["humidity"] = hum;

dht11["temperature"] = temp;

// soilMoisture section

JsonObject soilMoisture = doc.createNestedObject("soilMoisture");

soilMoisture["soilM"] = soil;

String jsonString;

serializeJson(doc, jsonString);

Serial.println("JSON Payload: " + jsonString);

int httpResponseCode = http.PUT(jsonString);

Serial.print("Firebase Current Data Response Code: ");

Serial.println(httpResponseCode);

if (httpResponseCode > 0) {

String response = http.getString();

Serial.println("Response: " + response);

} else {

Serial.println("Error in HTTP request for current data");

}

http.end();

// Now upload the same data to history

uploadToHistory();

}

void uploadToHistory() {

HTTPClient http;

// Create a unique timestamp for this record

String timestamp = getTimestamp();

String epochTimestamp = getEpochTimestamp();

// Upload to history using POST (this will create new entries without overwriting)

String historyUrl = String(firebaseHost) + "/history.json?auth=" + firebaseAuth;

Serial.println("Uploading to history: " + historyUrl);

http.begin(historyUrl);

http.addHeader("Content-Type", "application/json");

// Create JSON with timestamp and all sensor data DIRECTLY (no nested sections)

StaticJsonDocument<1024> historyDoc;

// Add timestamps

historyDoc["timestamp"] = timestamp;

historyDoc["timestamp_epoch"] = epochTimestamp;

historyDoc["readable_time"] = getCurrentDateTime();

// Add sensor values directly (no NPK, dht11, soilMoisture sections)

historyDoc["N"] = nitrogen;

historyDoc["P"] = phosphorus;

historyDoc["K"] = potassium;

historyDoc["PH"] = ph;

historyDoc["humidity"] = hum;

historyDoc["temperature"] = temp;

historyDoc["soilM"] = soil;

String historyJsonString;

serializeJson(historyDoc, historyJsonString);

Serial.println("History JSON Payload: " + historyJsonString);

int historyResponseCode = http.POST(historyJsonString);

Serial.print("Firebase History Response Code: ");

Serial.println(historyResponseCode);

if (historyResponseCode > 0) {

String response = http.getString();

Serial.println("History Response: " + response);

} else {

Serial.println("Error in HTTP request for history");

}

http.end();

// Also upload to daily stats

uploadToDailyStats();

}

void uploadToDailyStats() {

HTTPClient http;

// Get current date

String currentDate = getCurrentDate();

String currentDateTime = getCurrentDateTime();

// Upload to daily_stats with date as key (overwrite daily summary)

String dailyStatsUrl = String(firebaseHost) + "/daily_stats/" + currentDate + ".json?auth=" + firebaseAuth;

Serial.println("Uploading to daily stats: " + dailyStatsUrl);

http.begin(dailyStatsUrl);

http.addHeader("Content-Type", "application/json");

// Create JSON with last_ prefixed values for daily stats

StaticJsonDocument<1024> dailyDoc;

// Add last_ prefixed sensor values

dailyDoc["last_N"] = nitrogen;

dailyDoc["last_P"] = phosphorus;

dailyDoc["last_K"] = potassium;

dailyDoc["last_PH"] = ph;

dailyDoc["last_humidity"] = hum;

dailyDoc["last_temperature"] = temp;

dailyDoc["last_soilM"] = soil;

// Add last updated timestamp

dailyDoc["last_updated"] = currentDateTime;

String dailyJsonString;

serializeJson(dailyDoc, dailyJsonString);

Serial.println("Daily Stats JSON Payload: " + dailyJsonString);

int dailyResponseCode = http.PUT(dailyJsonString);

Serial.print("Firebase Daily Stats Response Code: ");

Serial.println(dailyResponseCode);

if (dailyResponseCode > 0) {

String response = http.getString();

Serial.println("Daily Stats Response: " + response);

} else {

Serial.println("Error in HTTP request for daily stats");

}

http.end();

}

void loop() {

// Process new data when received

if (newData) {

newData = false;

parseReceivedData();

}

// Upload data every 5 seconds

if (millis() - lastUploadTime >= uploadInterval) {

lastUploadTime = millis();

Serial.println("Uploading data to Firebase...");

i2cBusy = true;

uploadToFirebase();

i2cBusy = false;

}

// Small delay to prevent overwhelming the loop

delay(100);

}